Reexam

Indholdsfortegnelse

Reexam	1

Question 1. CMOS network	1

a.	Sketch the network	1

b.	Design the size of the transistors such that tpdr=tpdf	4

c.	Convert the circuit to its RC model. Then estimate tpd using elmore delay model.  R=10kΩ	6

d.	Simulating in LTSpice.	7

e.	Evaluate static and dynamic power consumptions for your circuit in LTspice. For dynamic power change the input from ABCD=0110 to ABCD=1101	8

Question 2. Sketch the implementation of a 2 input NAND using dynamic and static logic and compare them in terms of speed and size.	9

Question 3. Sketch the circuit of a pulse generator and describe how it can be used to make a circuit sensitive to both rise edge and fall edge of the clock. (Dual edge triggered).	11

11

Question 4 VHDL code	12

a.	Write a VHDL code for implementing the following truth table	12

12

Question 1. CMOS network

Y=A·B+D+C+DA

Sketch the network

Let me first start with the nmos network

Yn=Y=A·B+D+C+DA

Yn=AB+AD+C+AD

Yn=AB+AD+C

For the pull down network I need the function but knowing, that the inputs are gonna get inverted.

Y=YP=A·B+D+C+DA=AB+AD+C+AD=AB+AD+C

The cmos network:

Design the size of the transistors such that tpdr=tpdf

Wmin=200nm

L=60nm

μn=2μp

VDD=1V

As the mobility can be described as resistance in the paths the voltage can go through the network with, I know that:

R=UI

And I know that the current through a transistor can be described as

And as I know, that the current has a linear factor of μ I know that the resistance in a pmos is greater than the resistance in a nmos by a factor of two.

So I say that the resistance for a unit width for

A nmos=R

A pmos=2R

When looking at the paths I want to make the path a unit resistance. I also know that the current has a linear factor of Width two. With these things in mind I find the width for every transistor.

My findings:

================================================================================

Convert the circuit to its RC model. Then estimate tpd using elmore delay model.  R=10kΩ

C=0,1f

Cout=20fF

The mosfets RC equivalents:

Estimating for rise and fall should equal the same, as that’s what I designed the transistors for.

I look at the worst fall time and estimate the tpd from that. The worst case is when either B og D is 1 but not both and A=1, C=0.

I look at when A has been open for a long time, and B switching from 0 -> 1

1000→1100

I realised I have made a flaw in my CMOS network. The VDD can go straight through CAB

If I hadn’t made a mistake in my network I would have:

- Simplified the circuit.

- Solved for the time constant tau by:

τ=R1·C1+R1+R2·C2+…R1+…+Rn·Cn

With the sum of resistances being the ones to go through to get to the capacitor.

Simulating in LTSpice.

Simulating the circuit doesn’t make sense at this point as I know it will fail.

But I would have done as I’ve previously done in this circuit:

And I would have calculated the tpdf by looking at a time when a change in inputs from 0 -> 0,5*VDD would’ve resulted in a fall, and then look at, when the output got to 0,5*VDD. Then subtracting the first time from the second.

Same procedure for the tpdr.

tpd would then have been:

tpd=tpdf+tpdr2

My measurements would have looked a little like this.

If I were to change the widths of the transistors to compensate if the rise wouldn’t equal the fall propagation delay, I would have changed the width of the transistors corresponding to the change in inputs.

Evaluate static and dynamic power consumptions for your circuit in LTspice. For dynamic power change the input from ABCD=0110 to ABCD=1101

For the static power I would’ve looked at the average voltage over a period of the inputs changing (ABCD = 0000 -> 1111). I would’ve then timed it by the average current running from the VDD, from the same current (Real components have leakage currents), and then made a parameter for that.

The measurement would’ve been something like:

.meas pstatic  AVGVDD·AVGIVDD

With my circuit only running one period.

For the dynamic power consumption I would then look at the graph of the leakage current and:

- Find the ∆t in which the leakage current spiked the most.

- Multiplying them together with a target over that period.

It would’ve looked something like this:

.meas pdynamic VDD·IDD at=t1 target=t1+2·tpd

Assuming that the tpd would be time symmetric around 0 -> 0,5,  0,5 -> 1.

Question 2. Sketch the implementation of a 2 input NAND using dynamic and static logic and compare them in terms of speed and size.

Y=AB=A+B

My static implementation:

For the pull down network:

Yn=AB

For the pull up network:

Yp=A+B

With their inputs being inverted.

The comparison.

Static over dynamic:

- The static experiences lower delay when starting up:

  - The static doesn’t wait one clk cycle to be active. The dynamic does

- When A, B=1→0 at the same time period, the static will have a fast speed:

  - Two paths to VDD decreases the delay. The dynamic both have to wait for the clock, and has only 1 path two VDD.

- The size of the static is lower than the size of the static.

  - The static uses 2 pmoses, the dynamic uses 1. Designing for tpdf=tpdr then:

  - The static would have 2·2·Wmin+2·2Wmin=8Wmin worth of transistors

  - The dynamic would have 3·3·Wmin+2·Wmin=11Wmin worth of transistors

- Dynamic circuits require to run a setup phase before the output can be valid.

Question 3. Sketch the circuit of a pulse generator and describe how it can be used to make a circuit sensitive to both rise edge and fall edge of the clock. (Dual edge triggered).

Assuming all wires are 0 at t0, this is how the output would go.

t1:CLK→HIGH   -	a = 0, b = 1, out = 0 -> 1.

t2:CLK→LOW    -	a=0→ 1,   b=1-> 0,   out = 1→ 0→ 1

t3:CLK→HIGH  -	a=1→0,   b=0→1,       out=1→0→1

This is due to the time in which the signals are switching.

As signal a experiences some propagation delay, there will be a time period, in which both a=b>0,5V, this is where the signal will be 0.

When the signals has changed to their supposed values, the output will then again be 1.

It would look something like this:

Denote my use of “Something” as not the actual signal.

Question 4 VHDL code

- Write a VHDL code for implementing the following truth table

There is an error in the truth table, therefore row number 3 will be ignored.

Let me write KMAPs for the two bits.

How I do my implementation in VHDL is by the behavioral logic.

So by simplification the function can be written as conditional assigments:

a0=i2 when en=1 else'Z'

a1=0 when en=1 else'Z'

My declaration and architecture:

For the test bench I use a test bench generator just to get the template of the code that’s required.

It just ads my logic as a component, assign the signals and starts a process for me.

The template:

library IEEE;

use IEEE.Std_logic_1164.all;

use IEEE.Numeric_Std.all;

entity logik_tb is

end;

architecture bench of logik_tb is

component logik

Port ( i : in STD_LOGIC_VECTOR (2 downto 0);

a : out STD_LOGIC_VECTOR (1 downto 0);

en : in STD_LOGIC);

end component;

signal i: STD_LOGIC_VECTOR (2 downto 0);

signal a: STD_LOGIC_VECTOR (1 downto 0);

signal en: STD_LOGIC;

begin

uut: logik port map ( i  => i,

a  => a,

en => en );

stimulus: process

begin

wait;

end process;

end;

I then just assign the signals through 0000 -> 1111 and look at the output signal for every one

Steps then:

- “Run synthesis”, making sure that the circuit compiles.

- “Run simulation”

- Look at the output of a1, a0 and making sure, that it’s the result of the right inputs.

Note:

I’m using an Mac with an ARM system running the M chips. I’m interpretting linux to get Vivado to run. This comes with the downside being, that I cannot use testbench. Doing lab however I just used the ZYBO board for finding the true values, and that worked fine.