Eksamen 2022

Af Jesper Graungaard Bertelsen, AU-ID: au689481

Indholdsfortegnelse

Eksamen 2022	1

Teori	3

Fourier serie representation:	3

Frekvenser og deres representation	3

Periodisk:	3

Memoryless	4

Kausult	4

Stabilitet	4

Time variance:	5

Linearitet	5

Opgave 1	7

a.	Determine whether or not each of the following signals is periodic. Justify all your answers with necessary calculations. If it is periodic, determine the fundamental period.	7

b.	Determine whether or not the following systems are memoryless, causal, stable, time-invariant, and/or linear.	9

Memoryless	9

Causal	9

Stability:	10

Time invariance:	10

Linearity	11

Opgave 2. Fourier representation	12

Opgave 3: LTI system beskrevet ud fra differentialligning	17

a.	Determine the frequency response of the system.	17

b.	Sketch the bode plot of the system.	18

c.	Determine the output y1t when the input is:	18

Opgave 4. Discrete time LTI system with impulse response.	20

a.	Consider a discrete time LTI system with the impulse response hn=2un-un-5 and inputs signal…	20

b.	Consider a CTLTI system with the impulse response	21

c.	23

Opgave 3	29

Opgave 4	29

Teori

- Fourier serie representation:

Frekvenser og deres representation

ω0=π2 :

e-π2jk=cos-π2k+jsin-π2k

cos-π2k=0 for odd  1 for even divisible 4=0-1 for even divisible 4=2

sin-π2k=-1 for odd divisible 4=11 for odd divisble 4=3 0 for even

Derfor kan jeg splitte den exponentielle funktion op i:

e-π2jk=  1,  k % 4=0-j,  k % 4=1-1,  k % 4=2    j,  k % 4=3

ω0=π : 
e-jkπ=cos-kπ+jsin-kπ=-1 for odd1 for even
Kun reele dele tilbage. 
e-jkπ=-1,  for odd    1,  for even

- Periodisk:

xt=xt+T

For

xt=ft1+gt2
Gælder der, at funktionen kun er periodisk, hvis
T1T2=rationalt tal,  ( Altså ikke noget med pi i ).

1π2π=12:        rationalt tal 
22π=1π:   irrationalt tal

- Memoryless

Hvis systemet hverken ser på tidligere eller fremtidlige værdier, men kun ser øjebliksbilledet
yt=xt

- Kausult

Hvis for en hvilken som helst tid før start, at outputtet er 0, så er systemet kausult. 
yt=0,    t<0

- Stabilitet

Hvis der for hver tid t gælder, at

-B<xt<B,     for alle t

Og der da gælder at 
-N<yt<N,     for alle t
Så er systemet stabilt.

Eks. 
yt=t·xt 
Grænsen lægger jeg til: 
-B<xt<B
Da vil 
-t·B<yt<t·B    ikke være endelige for alle t'er
Ikke være endelig for alle t’er, da t kan gå mod uendelig, og outputtet vil derfor også gå mod uendelig. Da vil systemet ikke være stabilt.

Eks.
yt=x2t 
Grænsen lægger jeg til 
-B<xt<B,    for alle t
Og da vil 
-B2<yt<B2,    for alle t

Systemets output kan da beskrives som endeligt for alle t og dermed er systemet stabilt.

- Time variance:

https://en.wikipedia.org/wiki/Time-invariant_system

Systemet er tids invariant hvis et delay i inputtet er det samme som et delay i outputtet.

yt=xt
Input delay:
y1t=xdt-t0 
Output delay: 
y2t=xt-t0 
Derfor er systemet tids invariant, da y1=y2

Tids variant:
yt=txt 
Input delay: 
y1t=t·xdt-t0 
Output delay:
y2t=t-t0·xt-t0

Da y1≠y2 gælder der at systemet da er tids variant.

- Linearitet

Lidt samme smøre som for tids invarians: 
Hvis der gælder, at en ændring i amplitude foretaget af en faktor til inputtet resulterer i den samme faktor på outputtet, så er systemet lineært.

yt=xt 
En faktor på inputtet: 
y1t=axt 
En faktor på outputtet: 
ayt= axt 
Da y1t=ayt så er systemet lineært.

Ikke lineært
yt=xt2 
En ændring i inputtet: 
y1t=a·xta·xt=a2xt2 
Samme ændring nu på outputtet: 
ayt=axt2 
da y1t≠ayt så er systemet ikke lineært.

Opgave 1

Determine whether or not each of the following signals is periodic. Justify all your answers with necessary calculations. If it is periodic, determine the fundamental period.

(Computer plots are neither needed nor accepted as an answer.)

For peridocity the function must fullfill that:

xt=xt+T,

Where T is the fundamental period.

- xt=3cos3t+π3 is periodic as it’s a regular cos only with amplitude, frequency and phase.

A cosx has a period of 2π. 
xt=xt+T,       when 3t=2π 
t=23π. 
Thus, T is just a multiple of this period added to the function. 
T=23π

- xt=3·cos23t+π3 is a more dificult function to find the periodicity for.

But I can’t use trigonometric identities to describe it in another way.

xt=3·1+cos2·3t+2π32

xt=32+32·cos6t+23π

Which also follows the formula of a standard cos function, just now with a dc element.

It’s periodic when 
6t=2π=>T 
t=26π=13π=T

Thus this function is periodic with a periodicity of 13π

- xt=cosπ2t+cos12t might not be a periodic function as one frequency is of rational numbers and one irrational.

Researching it, it then actually shows, that this is a non-periodic function. 
If it wouldn’t have been, then

T1:2π=π2t,  t=4=T1 
T2:2π=12t,  t=4π=T2

Then the lowest common denominator will be the fundamental frequency. 
For this one, they already have common denominator, but as one of them is irrational, and one rational, this doesn’t mean, the period is T=1+π2. Otherwise, it would have been.

- xt=ejπt, I can already tell, that this function is periodic as this is the imaginary circle function. It has it’s period at 2π=πt=>T.   t=2=T

But to further show this, I can’t expand the function. 
xt=cosπt+j·sinπt 
as can be seen, they have the same frequency but different amplitude and phase, but these to doesn’t matter. 
What matters is that the trigonometric functions are periodic in 2pi.

T:2π=πt,   t=2=T.

=========================================================================

========================================================================

Efter fortolkning 
Reglen for hvornår summen af to funktioner er periodisk hedder, at forholdet 
T1T2 skal være rationelt for at summen af funktionerne er periodisk.

For mit tilfælde i 3’eren, så havde jeg T1T2=44π=1π, og da π er irrationel, så gælder kriteriet ikke her.

Determine whether or not the following systems are memoryless, causal, stable, time-invariant, and/or linear.

Memoryless

yt=xtxt+2 
As this function takes not only current but future values, then this system is not memoryless.

yn=maxxnxn+1 
This function with a reference at n = 0, takes in either a present value or a future value. As its not only a view into the present input, the system is said to be with memory.

Causal

Causality comes from an output being 0 for every t < 0. This can be translated into a description of a system that only response to present or past values.

yt=xtxt+2 
This system tries to predict the future inputs and is therefore regarded as a non-causal system.

yn=maxxnxn+1 
This system sends through a possible prediction for its output, therefore making this system non-causal.

Stability:

If the input is bounded to a finite number for any t, and the output then is bounded to a finite number for any of these inputs, then the system is stable.

Inputs: 
-B<xt<B,    for all t

yt=xtxt+2

-B·B<yt<B·B
B2<yt<B2
Then the system can be described within a finite boundary for all t, which makes this system stable.

yn=maxxnxn+1 
For this the boundary will be the same for either possible values, and as it’s just the input, then

-B<yn<B

Which is bounded within finite values, therefore the system is stable.

Time invariance:

If a delay at the inputs is equal to a delay at the outputs, then the system is time-invariant.

yt=xtxt+2

y1t=xt-t0xt-t0+2  <-delay at input 
yt-t0=xt-t0xt-t0+2  <-delay at output

Since y1t=yt-t0 then the system is time invariant.

yn=maxxnxn+1 
y1n=maxxn-noxn-n0+1  <-delay at input 
yn-n0=maxxn-noxn-n0+1 <-delay at output

Since y1n=yn-n0 then the system is time invariant.

Linearity

Now for the last property. This tells if a factor times the inputs is the same as the output being timed that same fraction.

yt=xtxt+2

y1t=axtaxt+2=a2xtxt+2 <- the input transformed

a·yt=axtxt+2 <-the output transformed

Since y1t≠ayt then this system is not linear.

yn=maxxnxn+1 
y1n=maxaxnaxn+1 <-input transformed

ayn=a·maxxnxn+1 <-output transformed

As y1n=ayn then the system is linear.

The properties are then

===========================================================================

|  | Memoryless | causal | Stable | TI | Linear |
| --- | --- | --- | --- | --- | --- |
| yt=xtxt+2 | No | No | Yes | Yes | No |
| yn=maxxnxn+1 | No | No | Yes | Yes | Yes |

===========================================================================

Opgave 2. Fourier representation

Determine the Fourier series representation of the following signal with period T = 4.

xt=-t,    for 0≤t<1  1,   for 1≤t<2  0,   for 2≤t<4

I found the analysis equation, which I will use to find
 the coefficients of this system.

ω0=2πT=2π4=12π

The integral I can split into the two non zero parts.

ak=1T01-te-jk·12πt+1T12e-jk12πt

ak=-1T01te-jk·12πt+1T12e-jk12πt

The first one needs integration by parts.

I solved it myself, to check the result of wolfram, and our results matched.

Now the second integral is easy. 
12e-jk12πt=1-jk12πe-jk12πt12=-1jk12πe-jkπ--1jk12πe-jkπ2=1jk·π2·e-jkπ2-e-jkπ

I then find:

ak=14·1π2j2k2·2πjke-π2jk-4+4e-π2jk+1jk·π2·e-jkπ2-e-jkπ

ak=14·-1·1π2k2·2πjke-π2jk-4+4e-π2jk+1jk·π2·e-jkπ2-e-jkπ

Now let me check the exponential functions for zeros and values.

e-π2jk=cos-π2k+jsin-π2k

cos-π2k=0 for odd  1 for even divisible 4=0-1 for even divisible 4=2

sin-π2k=-1 for odd divisible 4=11 for odd divisble 4=3 0 for even

I can then describe the exponential function in terms of even or odd numbers.

e-π2jk=-j   for odd % 4=1   j   for odd % 4=3   1  for even % 4=0-1 for even % 4=2

Now for my next exponential.

e-jkπ=cos-kπ+jsin-kπ=-1 for odd1 for even

Let me simplify the equation, such that 
c= e-π2jk

b= e-jkπ

ak=14·-1·1π2k2·2πjkc-4+4c+1jk·π2·c-b
ak=-2πjkc-4+4c4π2k2+c-b4·jk·π2

ak=0,5πjkc-1+cπ2k2+c-b2jkπ
ak=0,5πjkc-1+cπ2k2+-1c-b2kπj

ak=0,5πjkc-1+cπ2k2+b-c2kπj

So let me describe the coefficients for odd % 4 = 1.   [1, 5, 9, 13, 17, …]

ak=0,5πjk-j-1+-jπ2k2+-j--14·jk·π2

ak=-0,5πj2k-1-jπ2k2+-j+14·jk·π2jj

ak=0,5πk-1-iπ2k2---1+i4·k·π2
ak=0,5πk-1-iπ2k2-1+i4·k·π2

Now for real / imaginary:

ak=0,5πk-1π2k2-12·k·π-j·12·k·π+1π2k2

ak=πk-22π2k2-k·π2·k2·π2-j·k·π2·k2·π2+22·π2k2

ak=πk-2-kπ2π2k2-j·k·π+22·k2·π2

a1k≔-22π2k2-j·k·π+22·k2·π2

Figure 1: Python brute force results

a11=-0,2604761 j-0,1013212

a15=-0,03588384 j-0,004052847

a19=-0,01893476 j-0,001250879

They are equal. Now for the next odd.

ak=0,5πjkj-1+jπ2k2+-1-j2kπj

ak=-10,5πk-1+jπ2k2+-j--12kπ

Common denominator:

ak=-πk-2+2j2π2k2+-jkπ+kπ2k2π2

Figure 2: Brute force in python

Gathering real / imaginary parts: 
ak=-kπ-2+kπ2π2k2+j2-kπ2k2π2

a2k≔-22π2k2+j2-k·π2π2k2

a23=-0,04179374 j-0,01125791 
a27=-0,02066864 j-0,002067779 
a211=-0,01363127 j-8,373652·10-4 
They are equal.

Now for the even % 4 = 0:

ak=0,5πjk·1-1+1π2k2+1-12kπj

Parts go out, im left with:

ak=0,5πjkπ2k2

a3k≔12·π·kj

Figure 3: Brute force in python

So this one is purely imaginary: 
a34=0,03978874 j 
a38=0,01989437 j 
a312=0,01326291 j

They are the same. 
Now for the final one, the even % 4 = 2:

ak=0,5πjk-1-1+-1π2k2+1--12kπj

ak=-0,5πjk-2π2k2+22kπj
Common denominator
ak=-πjk-42π2k2+2kπ2k2π2j
ak=-42π2k2+2kπ-πk2k2π2j
a4k≔-2π2k2+12·k·πj

a42=0,07957747 j-0,05066059 
a46=0,02652582 j-0,005628955 
a410=0,01591549 j-0,002026424

Figure 4: Python brute force

They are equal.

Now I can describe ak purely by: 
ak=a1 for k % 4=1a4 for k % 4=2a2 for k % 4=3a3 for k % 4=0
If I had first realized, that the even term also depended on is divisible, then I would have made the functions in chronological order, but when I realized, I had already done some work.

===========================================================================

ak=-22π2k2-k·π+22·k2·π2j,   k % 4=1 -2π2k2+12·k·πj,  k % 4=2 -22π2k2+2-k·π2π2k2j,  k % 4=3                      12·π·kj,  k % 4=0

Which are the coefficients that can be used to recreate the signal

===========================================================================

I tried recreating this plot, but it didn’t seem like it could handle the large transistion that this signal has… not with the amount of time points ( 2000 ) and amount of coefficients ( 480 ) I had. Even though this required calculation of xt 2000 * 480 = 960.000 times! 
I was worried for my pc.

Opgave 3: LTI system beskrevet ud fra differentialligning

A linear time-invariant ( LTI ) system is described by the following differential equation

y+3y=x

Where x represent the input signal and y represent the output signal.

- Determine the frequency response of the system.

Hjω=YjωXjω

I chose to use laplace transform to describe this and set every initial condition to zero, thus making a transform of a differential

dydt→Ls·Yjω-0

Laplace on the system.
y+3y=x→LsYs+3Ys=Xs

Ys·3+s=Xs
Hs=YsXs=13+s

As s = jω

=================
Hjω=YjωXjω=13+jω 
=================

Sketch the bode plot of the system.

Sketched in terms of 20·log10absHjω from

ω=0…10000

Determine the output y1t when the input is:

xt=e-t,      t≥0             0,    otherwise

yt=xt⋆ht
But also 
Yjω=Xjω·Hjω

Yjω→L-1yt

And it’s that approach I will be doing.

xt=ut·e-t

Using sympy just to make sure, that it isn’t to different than just

xt=e-t,  for all t

It wasn’t.

For eat follows that the ROC is when 
Res>Rea F
For this one, as sympy returned for it’s second output, the real part must be larger than 
-a   or in this case  -1

Xjω=1s+1,     Res>-1

Yjω=Xjω·Hjω=1s+1·13+s=1s2+4s+3

Now for the inverse, I will do partial fraction on it.

My steps

b=1

a=1, 4, 3

Ys= -12s+3+12s+1

Now following the linearity principle.

ay1t+by2t→L aY1s+bY2s

a= -12,   b=12

Ys= a·1s+3+b1s+1

yt= ae-3t+be-t

And as the system is causal as well, the total description should be able to be written as:

=================================
yt=12·ut·e-t-e-3t 
or

yt=12·e-t-e-3t,      t≥0                                       0,      otherwise  
=================================

Opgave 4. Discrete time LTI system with impulse response.

- Consider a discrete time LTI system with the impulse response hn=2un-un-5 and inputs signal…

xn=un-un-3

- Sketch the signals h[n] and x[n]

- Sketch the output signal yn for the input xn.

hn=2, 2, 2, 2, 2, 0, 0

xn=1, 1, 1, 0, 0

A trick is to transpose xTh and calculating the sum alongside the right diagonal.

yn=2, 4, 6, 6, 6, 4, 2, 0, 0

Making sure with numpy.convole:

Which is the exact same that I got.

Consider a CTLTI system with the impulse response

ht=ut-ut-5

The following input signal is considered:

xt=ut-ut-3

- Sketch the signals ht and xt

So same procedure as the discrete time but a little different. For this first sketch it won’t make a difference.

- Sketch the output signal yt for the input xt

Now instead of the convolution sum in discrete, we now need to do the convolution integral.

Just with discrete, the response has been reversed, now in time.

Let me look at the regions for convolution.

t1=0,   t2=t

Area underneath is:

0t1·1=t

t1=0,  t2=3

Area underneath is:

031·1=3

t1=t-5,   t2=3

Area underneath is:

t-531·1=3-t-5=8-t

And the rest is 0.

Now I can describe

yt=t,    3>t>03,  5>t>38-t,  8>t>50,  otherwise

Opgave 5

- A continuous-time causal LTI system is characterized by the differential equation,

y-3y-4y=x-x

where x(t) represents the input signal and y(t) represents the output signal.

- Find H(s)

I use laplace transformation on the system and assumes, that the system is causal and thus at rest at start. With that I will have no initial slope, nor any initial accelerations.

y=s2Ys-sy0-y'0=s2Ys

y=sYs+y0                    =sYs

And the same for X.

s2Ys-3sYs-4Ys=sXs-Xs
Ys·s2-3s-4=Xs·s-1

=================
Hs=YsXs=s-1s2-3s-4 
=================

- Specify the region of convergence ROC, poles and zeros of H(s) on a plot.

I could do this multiple ways. For simplicity the partial fraction would probably be the easiest approach

I will use scipy to do the partial fraction.

Hs=25s+1+35s-4

And if the system is linear, as I suspect, then I have a ROC at Rx1∩Rx2. The common regions for the two regions of convergence.

And for linearity, the functions can be written as

Hs=25·1s+1+35·1s-4

Hs=ROC25·1s+1+ROC35·1s-4

Hs=25·ROC1s+1+35·ROC1s-4

And so, I have 
ROCHs=Res>-1∩Res>4

Thus the convergence occurs, when the real part of s is larger than 4. 

For poles and zeros, I will be using scipy to find those. scipy.signal.tf2zpkb, a
And I get.

Zeros:1+0j 
Poles:4+0j, -1+0j 
DC=1

Sketch the corresponding region of convergence in each pole zero plot below, considering the statements about x(t).

- x(t) is absolutely integral.

- x(t) = 0,   	t < -2

xt=xtut+2

- x(t)e^3t is absolutely integrable. ( Hint property of shifting in s-domain. )

Script for brute forcing of coefficients and plotting:

Plot for nk=12				Plot for nk=24

Plot for nk=36				plot for nk=48

Okay I see them converging against a different signal than for what I solved for. Maybe it has to do with the fact, that I’m only taking the positive k’s, thus not making it symmetrical. 
For k = 0 I need to do something, as this is a singularity.

As the a0 acts as a dc value, the average part, I can just make sure that I don’t calculate for this, at instead insert the dc value. 
For the signal I had a triangle, and a square. 
A=12·1·1+1·1=32

And the average is then the area divided by the period.

DC=324=38

Final plot
nt=250 
nk=196

I didn’t want to converge to the signal I wanted. Maybe I am missing something.

I used chat and it still used k only for the positive values. 
Now trying this again with 
nt = 1000
nk = 96

It seems like it converges into this one.

Mirroring it kinda makes it look like the orignal, but with a slope instead of the discontinuity

My most energy required plot:

nt=2000 
nk=480

Opgave 3

Opgave 4